1.

| Hardware Property | Local Measured Value | Hardware Property | Local Measured Value |
| --- | --- | --- | --- |
| CPU Model | AMD Ryzen 5 8645HS | Base & Max Boost Clock | 4301.0000/409.6200 |
| Physical Hardware Cores | 6 | Logical Cores (SMT/Threads) | 2 |
| L1 Data / Instruction Cache | 192 KiB (6 instances) | L2 Cache (Per Core / Total) | 6 MiB (6 instances) |
| L3 Shared Last-Level Cache | 16 MiB (1 instance) | SIMD Vector Extensions | AVX2, AVX-512 (F, DQ, BW, VL, VNNI), FMA, SSE4.2 |

Q1.1:  Map your host machine's execution modes into Flynn's Taxonomy

SISD: A single CPU core processes a sequential stream of scalar instructions on single data elements

SIMD: Vector execution units apply a single instruction across packed data registers using AVX2/AVX-512 extensions

MIMD: Multiple independent physical cores and SMT threads (6 cores / 12 threads) concurrently execute distinct instruction streams on separate memory spaces

Q2

Threads  | Run 1 (s)  | Run 2 (s)  | Run 3 (s)  | Avg TN (s)   | Speedup SN   | Efficiency EN

1        | 7.3164     | 2.7487     | 2.6196     | 4.2282       | 1.00        x | 100.0      %

2        | 1.3195     | 1.3080     | 1.3101     | 1.3125       | 3.22        x | 161.1      %

4        | 0.6559     | 0.6546     | 0.6555     | 0.6553       | 6.45        x | 161.3      %

8        | 0.4366     | 0.4363     | 0.4334     | 0.4354       | 9.71        x | 121.4      %

16       | 0.4313     | 0.4309     | 0.4302     | 0.4308       | 9.82        x | 61.3       %

32       | 0.4328     | 0.4294     | 0.4322     | 0.4315       | 9.80        x | 30.6       %

Q2.1:

Inflection Points: Sub-linear saturation begins at $N > 6$ (SMT engagement), with speedup reversing at $N > 12$ (oversubscription).

Physical vs. Logical Cores: The CPU has 6 physical cores and 12 hardware threads. Beyond 6 threads, sibling threads compete for shared ALUs and L1/L2 caches rather than utilizing dedicated silicon.

Scheduler Overhead ($N > 12$): Spawning 16–32 threads exceeds the 12 hardware contexts, forcing the OS scheduler into aggressive time-slicing and high-latency context switches.

Thermal & Cache Saturation: Shared 16 MiB L3 cache thrashing and mobile package power (TDP) constraints reduce all-core boost clocks under sustained multi-thread load

Task 3

| Run | Measured Output | Error (10^7 - Act) |

| #1  | 1624476         | 8375524            |

| #2  | 1613413         | 8386587            |

| #3  | 1422377         | 8577623            |

| #4  | 1768207         | 8231793            |

| #5  | 1845279         | 8154721            |

| #6  | 1841239         | 8158761            |

| #7  | 1727716         | 8272284            |

| #8  | 1702777         | 8297223            |

| #9  | 1905136         | 8094864            |

| #10 | 2185486         | 7814514            |

Q 3.1:

Instruction Breakdown: A non-atomic increment (counter++) compiles to three machine instructions: Load (read value from memory into a register), Add (increment ALU register), and Store (write register back to memory).

Interleaving: If Thread A reads value $X$, and Thread B reads $X$ before Thread A finishes writing back, both threads calculate $X + 1$ independently in their private registers.

Lost Updates: Thread B overwrites Thread A's stored result with the identical value $X + 1$. Despite two operations executing, the counter only advances by 1, dropping updates and causing severe data corruption.

Q 3.2:

Unlocked: 86.48 ms (Final Count: 10000000)Locked:

3847.98 ms (Final Count: 10000000)

Task 4:

Parallel fraction ($p$): 0.98 (using warmed-up $T_1 = 2.68$ s, $T_2 = 1.31$ s to remove cold-start distortion)

Sequential fraction ($1 - p$): 0.02

Max Theoretical Speedup ($S_{max}$): 50.00x

Amdahl 64-Core Projection ($S_{64}$): 28.32x

Gustafson Scaled Speedup ($S_{Gustafson}$): 62.74x

Q4.1: Strong vs. Weak Scaling

Amdahl (Strong Scaling): Workload size is fixed. The constant sequential fraction ($1 - p$) dominates total runtime as parallel time shrinks, placing a hard ceiling on speedup ($S_{max} = 50\times$).

Gustafson (Weak Scaling): Workload scales with core count ($64\times$ larger problem on 64 cores). The massive parallel workload dilutes fixed sequential overhead, achieving near-linear scaling ($62.74\times$)